CONTENTS

Page

Introduction	3

1	Scope	4

2	Normative references	4

3	Terms and definitions	4

4	Items to be measured	4

4.1	Video quality	4

4.1.1	Frame rate and linearity	4

4.1.2	Latency	5

4.1.3	PSNR	5

4.1.4	Colour reproduction	5

4.1.5	Impairment/Blockiness	6

4.2	Audio quality	7

4.2.1	Echo cancellation	7

4.2.2	Distortion	7

4.2.3	Background noise	7

4.2.4	Frequency response	7

4.2.5	Dynamic range	7

4.3	Overall quality	7

4.3.1	Synchronisation of audio and video (lip sync)	7

4.3.2	Scalability	7

Annex A (Informative)	8

Annex B (normative)  Normalization for calculation of PSNR	9

INTERNATIONAL ELECTROTECHNICAL COMMISSION

____________

MULTIMEDIA SYSTEMS AND EQUIPMENT – QUALITY ASSESSMENT –

AUDIO-VIDEO CONFERENCING SYSTEMS

- FOREWORD

1)	The IEC (International Electrotechnical Commission) is a worldwide organization for standardization comprising all national electrotechnical committees (IEC National Committees). The object of the IEC is to promote international co-operation on all questions concerning standardization in the electrical and electronic fields. To this end and in addition to other activities, the IEC publishes International Standards. Their preparation is entrusted to technical committees; any IEC National Committee interested in the subject dealt with may participate in this preparatory work. International, governmental and non-governmental organizations liaising with the IEC also participate in this preparation. The IEC collaborates closely with the International Organization for Standardization (ISO) in accordance with conditions determined by agreement between the two organizations.

2)	The formal decisions or agreements of the IEC on technical matters express, as nearly as possible, an international consensus of opinion on the relevant subjects since each technical committee has representation from all interested National Committees.

3)	The documents produced have the form of recommendations for international use and are published in the form of standards, technical specifications, technical reports or guides and they are accepted by the National Committees in that sense.

- In order to promote international unification, IEC National Committees undertake to apply IEC International Standards transparently to the maximum extent possible in their national and regional standards. Any divergence between the IEC Standard and the corresponding national or regional standard shall be clearly indicated in the latter.

- The IEC provides no marking procedure to indicate its approval and cannot be rendered responsible for any equipment declared to be in conformity with one of its standards.

- Attention is drawn to the possibility that some of the elements of this international standard may be the subject of patent rights. The IEC shall not be held responsible for identifying any or all such patent rights.

International Standard IEC 6XXX has been prepared by IEC technical committee 100: Audio, Video and Multimedia Systems and Equipment.

The text of this standard is based on the following documents:

| FDIS | Report on voting |
| --- | --- |
| 100/XX/FDIS | 100/XX/RVD |

Full information on the voting for the approval of this standard can be found in the report on voting indicated in the above table.

This publication has been drafted in accordance with the ISO/IEC Directives, Part 3.

# Introduction

This proposal was performed as a stage 0 by a small group of experts from the former project team 61966 on colour measurement and management in multimedia systems and equipment as an expansion of the scope for PWI 61966-11: Quality assessment – Impaired video in network systems.  The reasons of this expansion has been briefly described in 100/AGM(London/PT 61966)13 of 24 March 1998.  It was further worked and presented as 100/AGM(PT 61966)67.

A study group on audio-video quality has been installed in the Institute of Image Electronics Engineers of Japan. The study group recognized market needs to work and to standardize “IEC 6xxxx: Multimedia systems and equipment – Quality Assessment – Audio-Video Teleconferencing Systems” as part of quality aspects of multimedia systems and equipment.

MULTIMEDIA SYSTEMS AND EQUIPMENT – QUALITY ASSESSMENT –

AUDIO-VIDEO CONFERENCING SYSTEMS

# Scope

This International Standard specifies items to be measured by objective methods, methods of measurement together with measuring conditions, processing of the measured data and forms to report acquired information for assessment of end-to-end quality of audio-video teleconferencing systems over worldwide digital networks, where pear-to-pear bandwidth is time variant and is not garranteed when it established.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this International Standard. For dated references, subsequent amendments to, or revisions of, any of these publications do not apply. However, parties to agreements based on this International Standard are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. For undated references, the latest edition of the normative document referred to applies. Members of IEC and ISO maintain registers of currently valid International Standards.

# Terms and definitions

3.1

teleconferencing system

network based digital system in which multiple media are in use for information exchange among participants in a virtual conference

3.2

vertual conference

meeting of a group of people who do not assemble to the same geographical place, but they exchange their views and opinions in use of multimedia logically connected each other.

3.3

latency

time required to send and receive a signal.

3.4

lineality

the number of video frames skipped at receiving end

3.5

PSNR

peak signal to noise ratio

# Items to be measured

## Video quality

### Frame rate and linearity

The original analogue video is encoded at 30 frame/s and transmitted to a client by either live or on-demand over the local area network.  The frame rate of received digital videos are measured.

Table – Streamed and received frame rates

| Product | Small size (160120) | Large size (320240) |
| --- | --- | --- |
| N | 4,95 fps | 4,97 fps |
| R | 14,71 fps | 12,51 fps |
| V | 28,01 fps | 28,55 fps |

NOTE – The product N performed in real time encoding and streaming, the other products steamed encoded and archived digital videos.

The linearity is also measured.  For all products, fairly good linearity are noted.

### Latency

The delay time in second from analogue video input to encoder and received digital video.

### PSNR

#### Method of measurement

The peak signal-to-noise ratio between a full reference image and a reproduced image define by the following equation shall be calculated.

PSNR=10 log102552MSEMSE=1(P2−P1+1)(M2−M1+1)(N2−N1+1)p=P1P2 m=M1M2 n=N1N2(d(p,m,n)−o(p,m,n))2

where d(p,m,n) and o(p,m,n) represent respectively degraded and original pixel value at frame p, row m and column n.

NOTE  PSNR requires a very high degree of normalisation to be used with confidence. The normalisation requires both spatial and temporal alignment as well as corrections for gain and offset. The normalisation method is described in annex B.

#### Report of the measured result

### Colour reproduction

#### Method of measurement

Under consideration. See also worked example in annex A.

#### Report of the measured results

The measured results shall be reported together with the conditions use in the measurements.

NOTE  Table shows examples of colour shift in Δu'v from the original NTSC colour bars directly displayed on the same equipment using cathode ray tubes.

Table – Average colour shifts of saturated colours

| Condition | Small size (160120) | Large size (320240) |
| --- | --- | --- |
| N | 0,025 | 0,012 |
| R | 0,042 | 0,015 |
| V | 0,029 | 0,009 |

### Impairment/Blockiness

## Audio quality

### Echo cancellation

### Distortion

### Background noise

### Frequency response

### Dynamic range

## Overall quality

### Synchronisation of audio and video (lip sync)

Received and recorded videos are analized to acquire delay between synchronised audio and video.

Table – Synchronisation

| Product | Small size (160120) | Large size (320240) |
| --- | --- | --- |
| N | 0,699 (s) | 0,818 (s) |
| R | 0,494 (s) | 0,469 (s) |
| V | 0,448 (s) | 0,446 (s) |

### Scalability

Autonomous function to tune frame rate dynamically depending on time variant available bandwidth between the server and the client logical link.

# Annex A
(Informative)

Video quality assessment (trial)

- A.1 Introduction

This annex is prepared just for information to report some of the preliminary results of measurement made in The Laboratory of Multimedia Systems, Faculty of Engineering, Chiba University, Japan.   Three different target products for application to desk-top personal computers are tested.  They are getting popular and are publicly available for evaluation.

- A.2 Configuration for measurement

Figure A.1 – General configuration for measuement

# Annex B
(normative)

Normalization for calculation of PSNR

- B.1

- B.2

- Bibliography

ITU-T/SG 9: Temporary document 50Rev 1, “PROPOSED DRAFT NEW RECOMMENDATION J.FULLREF” on Question 22/9, Geneva, 15 – 19 May 2000

Measuring quality in videoconferencing systems, Part number PC316, Intel Corporation (November 1997)

Criteria for product evaluation, NASA Desktop video expert center, National Aeronautics and Space Administration, Ames Research Center, Moffett Field, California (August 1997)

Quality aspects of computer-based video services, Norbert Gerfelder (Fraunhofer Institute for Computer Graphics, Darmstadt, Germany and Wolfgang Muller (Darmstadt Technical University), (Oct. 1995)